# WALL-MOUNTED STRING HARP SCULPTURE

Complete Technical Reference & Build Guide

Interactive Community Sound Installation

Document Version: 1.0

November 2025

# Chapter 1: Executive Summary

## 1.1 Project Overview

The Wall-Mounted String Harp is a large-scale interactive sound sculpture designed for public spaces. This 25-foot wide by 9-foot tall installation transforms a hallway wall into a playable musical instrument, inviting community members to explore sound through direct interaction.

## 1.2 Recommended Configuration

| Specification | Value |
| --- | --- |
| Total Strings | 144 (2 per note, 6 octaves) |
| String Spacing | 54.4mm (2.14 inches) |
| Musical Range | C2 to B7 (6 complete octaves) |
| Wall Dimensions | 25 feet wide × 9 feet tall |
| Total Depth | 14 inches (from wall to front) |
| Total Tension | 203,336 N (45,704 lbf) |
| Estimated Budget | $15,000 - $20,000 |
| Installation Time | 10-14 days (professional team) |

## 1.3 Key Features

- Wide string spacing enables precise playing by community members of all ages

- Natural chorus effect from doubled strings creates rich, shimmering sound

- Double-capo system allows 6 octaves from 7-foot strings

- Hybrid material selection optimizes sound quality across full range

- Birch plywood resonator with air gap amplifies sound naturally

- Manageable maintenance with quarterly tuning schedule

# Chapter 2: Design Philosophy & Rationale

## 2.1 Why 144 Strings?

The 144-string configuration (2 strings per note) represents an optimal balance between musical capability, playability, and practicality. This chapter explores the decision-making process that led to this recommendation.

### Configuration Comparison

| Aspect | 72 Strings | 144 Strings | 464 Strings |
| --- | --- | --- | --- |
| Spacing | 108mm (4.3") | 54mm (2.1") | 17mm (0.7") |
| Musical Range | 6 octaves | 6 octaves | 4 octaves |
| Playability | Excellent | Very Good | Difficult |
| Visual Density | Sparse | Balanced | Dense |
| Estimated Cost | $12,000 | $18,000 | $32,000 |

## 2.2 The Double-Capo Innovation

The double-capo system is the key innovation that enables this instrument to achieve 6 octaves of range while keeping all strings at a uniform 7-foot physical length.

How It Works:

- Each string has TWO adjustable capos along its length

- Lower capo defines bottom of playable zone (e.g., 189mm from soundbox)

- Upper capo defines top of playable zone (e.g., 1,944mm from soundbox)

- Only the section BETWEEN capos vibrates when struck

- Bass strings have capos far apart (1,755mm playable = low frequency)

- Treble strings have capos close together (600mm playable = high frequency)

Benefits:

- Strings remain physically thick and easy to handle throughout

- Playing zone always centered at comfortable chest-to-head height

- Achieves high pitches without dangerous string tensions

- Easy to retune entire instrument for different scales

# Chapter 3: String Physics & Materials

## 3.1 Fundamental Wave Equation

All string behavior follows the fundamental wave equation:

f = (1/2L) × √(T/μ)

Where:

- f = frequency in Hertz (Hz)

- L = vibrating length in meters (m)

- T = tension in Newtons (N)

- μ = linear mass density in kg/m

## 3.2 Why Different Materials Are Essential

Using a single material for all strings is physically impossible for a 6-octave range. Here's why:

The Problem with Single Materials:

If you use steel wire (1.0mm diameter) for all strings: Bass strings (C2 at 65Hz) work fine with 342N tension and 1.8m length. But treble strings (B7 at 3,951Hz) would require EITHER a 1.17-inch playable length (impossible to play with mallets) OR 138,549N tension (string would explode instantly).

### Material Selection Strategy

| Range | Material | Diameter | Density |
| --- | --- | --- | --- |
| Bass (C2-E3) | Phosphor Bronze Wound | 1.6-2.0mm | 8,000 kg/m³ |
| Mid (F3-B5) | Steel Plain | 1.0-1.2mm | 7,850 kg/m³ |
| Treble (C6-B7) | Fluorocarbon | 0.8-0.9mm | 1,780 kg/m³ |

### Material Properties & Benefits

Phosphor Bronze Wound (Bass):

- Construction: Steel core with bronze wire wound around it

- Sound: Rich, warm, powerful bass tone

- Visual: Substantial appearance reinforces bass character

- Cost: $2.00-2.50 per string

Steel Plain (Mid-range):

- Construction: Solid steel music wire

- Sound: Bright, clear, sustaining

- Properties: Minimal stretch, holds tune well

- Cost: $0.60 per string

Fluorocarbon (Treble):

- Construction: High-performance polymer (fishing line grade)

- Sound: Brilliant, singing tone

- Properties: UV resistant, weather resistant, low stretch

- Cost: $0.40 per string

# Chapter 4: Structural Design & Construction

## 4.1 Layered Construction System

The instrument uses a five-layer construction, each serving specific acoustic and structural functions:

| Layer | Material | Thickness | Distance from Wall | Purpose |
| --- | --- | --- | --- | --- |
| 1. Wall Mount | Steel/Wood | 50mm | 0mm | Structural attachment |
| 2. Air Gap | Empty space | 127mm | 50mm | Acoustic resonance |
| 3. Resonator | Birch plywood | 19mm | 177mm | Sound amplification |
| 4. String Zone | Strings/capos | 50mm | 196mm | Sound generation |
| 5. Protection (opt) | Polycarbonate | 12mm | 346mm | String protection |

## 4.2 Soundbox Specifications

- Dimensions: 7,830mm wide × 610mm tall × 300mm deep

- Construction: Marine plywood (18mm) with finger joints

- Internal bracing: Cross-braces every 600mm

- String anchors: 144 brass inserts with rosewood bridge pins

- Finish: Tung oil or water-based polyurethane

## 4.3 Top Bridge

- Dimensions: 7,830mm wide × 100mm tall × 80mm deep

- Material: Hard maple or oak (3-piece laminated)

- String contact: Precision V-grooves at 54.4mm spacing

- Groove depth: 5mm per groove

## 4.4 Resonator Panel

- Material: Baltic birch plywood, 19mm (13-ply construction)

- Dimensions: 7,830mm × 2,744mm (full wall coverage)

- Bracing: Diagonal spruce bracing (20mm × 40mm) every 400-600mm

- Finish: Thin shellac or lacquer (allows vibration)

- Air gap behind: 127mm (5 inches) for acoustic coupling

# Chapter 5: Double-Capo Tuning System

## 5.1 Capo Mechanical Specifications

Capo Body:

- Material: Anodized aluminum (6061-T6)

- Dimensions: 60mm wide × 25mm tall × 20mm deep

- Weight: Approximately 15g per capo

- Finish: Black or blue anodized

Pressure Pad:

- Material: Silicone rubber (Shore A 60-70 durometer)

- Dimensions: 50mm × 10mm × 5mm

- Purpose: Dampens string cleanly without buzzing

- Replacement interval: Every 2-3 years with heavy use

Mounting System:

- Type: T-slot aluminum extrusion (20mm × 20mm profile)

- Material: 6063-T5 aluminum, clear anodized

- Length: 2.5m per string (144 rails total)

- Operation: Captured T-nut allows smooth sliding, prevents rotation

Adjustment Mechanism:

- Type: Thumbscrew with locking nut

- Thread: M4 × 0.7mm pitch

- Material: Stainless steel 316

- Knob: Knurled aluminum, 15mm diameter

## 5.2 Tuning Procedure

- Position both capos at approximate locations for desired note

- Pluck string and measure frequency with electronic tuner

- Fine-tune by moving ONE capo: closer together = higher pitch, farther apart = lower pitch

- Moving capo 10mm typically changes pitch by 1-2 semitones

- Lock both capos when string is in tune (finger-tight plus 1/4 turn)

- Mark final positions with paint pen for easy retuning

Tuning Precision:

- Resolution: 1mm adjustment = approximately 0.2 semitones

- Stability: Capos should not drift more than ±0.5mm over 6 months

- Recommended retuning: Every 2-4 weeks depending on climate

# Chapter 6: Complete Materials List & Budget

## 6.1 String Materials

| Component | Quantity | Unit Cost | Subtotal |
| --- | --- | --- | --- |
| Phosphor Bronze Wound (bass) | 36 strings | $2.00 | $72 |
| Steel Plain (mid-range) | 60 strings | $0.60 | $36 |
| Fluorocarbon (treble) | 48 strings | $0.40 | $19 |
| STRING TOTAL | 144 strings | - | $127 |

## 6.2 Budget Summary

| Category | Cost Estimate |
| --- | --- |
| Strings (144 total) | $127 |
| Capo System (288 capos + rails) | $5,500 |
| Structural Components (frame, soundbox, bridge) | $3,200 |
| Resonator Panel (birch plywood) | $360 |
| Wall Mounting Hardware | $400 |
| Finishing Materials & Hardware | $800 |
| Tools & Equipment | $350 |
| Optional: Protective Layer (polycarbonate) | $4,000 |
| SUBTOTAL (without protection) | $10,737 |
| With 15% Contingency | $12,350 |
| With Protection + Contingency | $17,050 |
| Professional Installation (optional) | +$3,000-5,000 |

# Chapter 7: Installation Procedure

## 7.1 Installation Timeline

| Phase | Tasks | Duration |
| --- | --- | --- |
| Wall Prep | Locate studs, install mounts, verify load capacity | 1-2 days |
| Frame Assembly | Build frame, attach resonator, install capo rails | 2-3 days |
| Wall Mounting | Lift and secure frame (4-6 people required) | 1 day |
| String Installation | Install 144 strings, gradual tensioning, capo placement | 3-4 days |
| Tuning | Tune all strings, document capo positions | 2-3 days |
| TOTAL |  | 10-14 days |

## 7.2 String Installation Strategy

- Begin stringing from center and work outward to balance frame loads

- Initially tension all strings to 50% of target tension

- Allow frame to settle for 24-48 hours

- Gradually increase to 75% tension and settle again

- Final tensioning to 100%

- This gradual approach prevents frame warping and premature string breakage

# Chapter 8: Maintenance & Care

## 8.1 Maintenance Schedule

| Interval | Tasks |
| --- | --- |
| Daily | Visual inspection for broken strings, check loose capos, light dusting |
| Weekly | Spot-check tuning of 10-15 strings, tighten loose thumbscrews, clean high-touch areas |
| Monthly | Full tuning check, retune drifted strings, structural inspection, deep clean with alcohol |
| Quarterly | Complete retuning, replace damaged strings, inspect capo pads, check frame integrity |
| Annually | Replace 10% of strings, replace worn capo pads, full structural inspection, refinish wood |

## 8.2 Annual Operating Costs

- String replacement: 15-20 strings/year = $20-30

- Capo pad replacement: 20-30 pads/year = $60-90

- Cleaning supplies and touch-up materials = $40-60

Total estimated annual maintenance cost: $120-180

# Appendix A: Complete Technical Specifications

| Specification | Value / Description |
| --- | --- |
| Overall Dimensions | 7,830mm W × 2,744mm H × 356mm D (25.7' × 9' × 14") |
| Total Strings | 144 (72 notes × 2 strings per note) |
| String Spacing | 54.4mm (2.14 inches) center-to-center |
| Musical Range | C2 (65.41 Hz) to B7 (3,951.07 Hz) - 6 octaves |
| Full String Length | 2,134mm (7 feet) - uniform for all strings |
| Playable Zone Range | 600mm to 1,800mm (varies by pitch via double-capo) |
| Total String Tension | 203,336 N (45,704 lbf / 20,728 kgf) |
| Average Tension per String | 1,412 N (318 lbf / 144 kgf) |
| Total Capos | 288 (2 per string: upper and lower) |
| Frame Material | 40mm × 80mm aluminum extrusion, powder-coated |
| Resonator Material | Baltic birch plywood, 19mm, 13-ply |
| Soundbox Volume | Approximately 1.43 cubic meters |
| Estimated Weight | 180-220 kg (400-485 lbs) fully assembled |
| Wall Loading | Approximately 25-30 kg/m² (5-6 lbs/ft²) |
| Tuning System | Chromatic (12-tone equal temperament) |
| Recommended Installation | Indoor climate-controlled space, 50-60% RH, 65-75°F |

# Appendix B: Complete 144-String Specification Table

The following table shows the first 12 strings (one complete octave) as a representative sample. The complete specification for all 144 strings is available in the accompanying CSV file.

| String | Note | Freq (Hz) | Material | Dia (mm) | Tension (N) | Play Len (mm) | Capos (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C2 | 65.41 | Bronze Wound | 2.0 | 200 | 1,755 | 189-1,944 |
| 2 | C2 | 65.41 | Bronze Wound | 2.0 | 200 | 1,755 | 189-1,944 |
| 3 | C#2 | 69.30 | Bronze Wound | 2.0 | 200 | 1,655 | 239-1,894 |
| 4 | C#2 | 69.30 | Bronze Wound | 2.0 | 200 | 1,655 | 239-1,894 |
| ... | ... | ... | ... | ... | ... | ... | ... |

Note: Complete 144-string specification table with all calculated values is available in the accompanying digital files (CSV format).

# Document Revision History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | Nov 2025 | Initial comprehensive documentation release |

END OF DOCUMENT

For questions, updates, or technical support, please maintain this document as a living reference.